# GreenNotify - Golf Yardage Notification App

Application Specification Document v1.0

Date: 27 Oct 2026 | Target: Android APK (Kotlin) | Region: Bangkok Metro + Adjacent Provinces (33 Courses, 594 Greens) | Author: Spec Draft for Owner

# Table of Contents

1. Executive Summary
2. Product Vision & Goals
3. Scope
4. Target Users & Personas
5. Core Features
6. System Architecture
7. Data Model - Greens & Geofencing
8. Notification Trigger Logic (Core IP)
9. Yardage Calculation
10. Notification & UX Specification
11. Battery & Performance Requirements
12. UI/UX Flow
13. Tech Stack
14. Permissions & Privacy
15. Course Data - Bangkok Metro
16. API Integrations
17. Testing & Acceptance Criteria
18. Roadmap & Milestones
19. Risks & Mitigations
20. Appendix - Kotlin Snippets

# 1. Executive Summary

GreenNotify is an Android app that automatically notifies golfers of yardage to the green for the current hole, like a new message alert, without requiring the user to open the app or check GPS manually. The app uses green polygon geofencing (Bangkok Metro dataset), hybrid motion + geofence triggers, and shot detection to deliver a single, timely notification per shot position. Goal: Reduce battery consumption to <1% per hour while achieving <2% missed notifications.

# 2. Product Vision & Goals

Vision: "Caddie in your pocket - know your yardage before you even ask."

Business Goals:

Provide yardage alerts for 33 courses in Bangkok Metro at launch, expand to 300 Thailand-wide

Battery efficient: 36 holes on single charge

Zero manual interaction during round - fully hands-free

Accuracy: GPS yardage ±3 yards, green entry detection ±5m

# 3. Scope

IN SCOPE: Course selection, green polygon geofencing, yardage to front/center/back, pin position (future), shot detection via accelerometer, hybrid trigger, push-style notifications, history/log, offline mode.

OUT OF SCOPE (v1): Scorecard, social, swing analytics, live pin from superintendent (v2 via StrackaHoLo), iOS, wearable.

# 4. Target Users

Primary: Thai golfers aged 25-55 playing in Bangkok/Pattaya area, using Android, playing 2-4x per month. Secondary: Tourists, Caddies, Golf academies.

# 5. Core Features

F1 - Course & Hole Selection: User selects course (33 Bangkok Metro), selects starting hole. App loads green polygons for that course only (18 greens).

F2 - Smart Yardage Notification: One notification per ball position: Yardage to front/center/back + wind + club suggestion.

F3 - On-Green Detection: When inside green polygon, notify "On Green! X ft to pin".

F4 - Hole Auto-Advance: When user leaves green and enters next tee geofence, auto-advance.

F5 - Shot Tracking: Detect swings via accelerometer, log shot count per hole, distance per shot.

F6 - Offline Mode: Green polygons stored locally in Room/SQLite, no server needed on course.

F7 - Round History: Store round logs, yardage history, battery usage.

# 6. System Architecture

Architecture: MVVM + Clean Architecture. Presentation (Compose UI) -> ViewModel -> Repository -> Data Sources (Room DB for greens, FusedLocation, GeofencingClient, ActivityRecognition). Background: WorkManager + Foreground Service for round session.

Components:

GeofenceManager.kt - Manages 2 geofences per active hole only

ShotDetector.kt - Accelerometer listener, peak detection >2.5g

LocationManager.kt - FusedLocationProvider, adaptive accuracy

YardageEngine.kt - Haversine + polygon point-in-polygon (Ray casting)

NotificationManager.kt - Notification channels, vibration patterns

CourseRepository.kt - Loads GeoJSON -> Room

# 7. Data Model - Greens & Geofencing

Green Polygon Schema (GeoJSON):

{
  "course_name": "Alpine Golf Club",
  "province": "Bangkok",
  "hole": 7,
  "green_id": "ALP-G07",
  "type": "green",
  "area_m2": 587,
  "geometry": {"type": "Polygon", "coordinates": [[[lng, lat], ...]]}
}

Room Entities: Course(id, name, province, lat, lng), Green(id, courseId, hole, polygonJson, centerLat, centerLng, frontLat, frontLng, backLat, backLng), Pin(courseId, hole, lat, lng, date), RoundSession(id, courseId, startTime, batteryStart), Shot(id, roundId, hole, lat, lng, timestamp, club)

Geofence Definitions: Tee Geofence 120m radius, Green Geofence 45m radius (or polygon if available - use polygon for on-green, circle for pre-wake). Only active hole geofences registered at a time to save battery.

# 8. Notification Trigger Logic - Core IP

PROBLEM: Pure GPS geofence = 5-8%/hr battery. Pure interval = 20% miss rate.

SOLUTION: Hybrid 3-Stage Trigger:

Stage 1 - Passive Geofence (Hardware): GeofencingClient registers Tee + Green. OS uses cell/WiFi, not GPS. Cost 0.3%/hr. Dwell 3s.

Stage 2 - Shot Detection: On entering Tee geofence, start accelerometer @ 50Hz low-power. Detect swing spike >2.5g followed by 2s stillness. Mark shot.

Stage 3 - Adaptive Location: After shot, wake GPS HIGH_ACCURACY for 10s, get position, compute yardage, notify, then return to BALANCED_POWER. Adaptive interval: >200y = 15s, 100-200y = 8s, <100y = 5s, STILL = 60s.

Debounce Rule: Notify ONLY ONCE per position. Conditions: (1) Moved >15m from last notified position, AND (2) STILL for 3s (ActivityRecognition = STILL), AND (3) GPS accuracy <15m. Prevents spam.

State Machine: IDLE -> ON_TEE -> IN_PLAY (after Shot 1) -> APPROACH -> ON_GREEN -> HOLE_COMPLETE -> NEXT_TEE

Edge Cases: Cart driving through green - loitering delay filters. GPS drift - filter by accuracy. Lost signal - use last known location + fallback to green center.

# 9. Yardage Calculation

Haversine formula for distance. Front/Back: Compute closest and furthest points of green polygon from ball position, project onto green center line. Wind: OpenWeatherMap API cached per course. Display in yards (imperial default) + meters toggle. Accuracy target ±3y.

# 10. Notification & UX Specification

Notification Channels: Channel 1 - YARDAGE (high importance, vibration, sound). Channel 2 - HOLE_INFO (low).

Notification Format:

Title: "Hole 7 Par 4 - 142y to Green"

Body: "Center 142y | Front 128y | Back 154y | Wind NE 8km/h | Suggest 8 Iron"

Actions: [View Map] [Next Hole] [Mute]

Vibration: 200ms, pause 100ms, 200ms. Sound: short chime. On Green: longer vibration + "On Green! 12ft to pin"

Clarification: "After Shot 2" label means notification is FOR Shot 2, triggered BY Shot 1. User gets alert BEFORE playing next shot, not after.

# 11. Battery & Performance Requirements

Targets: Hybrid mode 0.8%/hr, Max 1.5%/hr. 36 holes on 80% charge. CPU <5% average. Memory <150MB. Cold start <1.5s. Offline - no network needed after course download.

# 12. UI/UX Flow

Screens: 1. Home - Select Course (search, province filter, nearby). 2. Round Setup - Choose starting hole, tee color, wind. 3. Round Active - Minimal UI: big yardage, hole number, map with green polygon. 4. Notification - System notification even when app backgrounded. 5. History - Round logs.

Flow: Open App -> Select Alpine Golf Club -> Press Start Round -> Put phone in pocket/cart -> Receive notifications per shot -> Finish -> Summary.

# 13. Tech Stack

Language: Kotlin 1.9, Jetpack Compose

Location: Google Play Services FusedLocationProviderClient, GeofencingClient, ActivityRecognitionClient

Storage: Room + SQLite, DataStore for preferences

Maps: Google Maps SDK or MapLibre (offline tiles)

Notifications: NotificationCompat, Foreground Service (type=location)

Build: Gradle, minSdk 26 (Android 8), targetSdk 34

Distribution: APK + AAB via Play Console Internal Testing

# 14. Permissions & Privacy

Required: ACCESS_FINE_LOCATION, ACCESS_BACKGROUND_LOCATION, POST_NOTIFICATIONS, ACTIVITY_RECOGNITION, FOREGROUND_SERVICE_LOCATION, VIBRATE, INTERNET (for weather). Explain in-app: "We need background location to notify yardage when phone is in pocket." Data stays on device, no tracking outside round.

# 15. Course Data - Bangkok Metro

Launch Courses (33): Bangkok (5), Pathum Thani (7), Samut Prakan (6), Chachoengsao (5), Nakhon Pathom (4), Ayutthaya (5). Each course: 18 greens, polygons from GeoJSON file bangkok_greens.geojson (594 features). Note: Current polygons are placeholder georeferenced ellipses around true course centers - replace with Stracka Golf Intelligence API polygons for production.

# 16. API Integrations

Stracka Golf Intelligence API - Green polygons, elevation (commercial). OpenWeatherMap - Wind. Google Maps - Tiles. Optional: StrackaHoLo for daily pin.

# 17. Testing & Acceptance Criteria

Test on 3 courses: Alpine, Lotus Valley, Nikanti - 18 holes each

Battery test: 18 holes <15% drain

Notification test: At least 90% of shots trigger 1 notification within 10s of arriving

No duplicate notifications for same position

Works offline with airplane mode + GPS only

App works with screen off

# 18. Roadmap

Phase 1 (v1.0 - 4 weeks): Bangkok 33 courses, hybrid trigger, yardage notification. Phase 2 (v1.1): Daily pin input, club suggestion AI. Phase 3 (v2.0): 300 courses Thailand-wide, scorecard, Apple Watch.

# 19. Risks

Risk: Android OEM kills background service (Xiaomi/Oppo) - Mitigation: Battery optimization exemption request, Foreground Service. Risk: GPS accuracy in trees - Mitigation: Kalman filter + polygon tolerance.

# 20. Appendix - Kotlin Snippets

Geofence creation:

val geofence = Geofence.Builder().setRequestId("GREEN_7").setCircularRegion(greenLat, greenLng, 45f).setExpirationDuration(Geofence.NEVER_EXPIRE).setTransitionTypes(Geofence.GEOFENCE_TRANSITION_ENTER or Geofence.GEOFENCE_TRANSITION_DWELL).setLoiteringDelay(3000).build()

Shot detection:

if (accelMagnitude > 2.5g && lastShotTime > 5s) { onShotDetected() }

Yardage:

fun haversine(lat1, lon1, lat2, lon2): Double { ... return distanceMeters * 1.09361 } // to yards